Describe the construction and operation of the following electrochemical cells:

a)	the dry cell, primary cell

b)	the lead-acid accumulator, secondary cell

c)	the fuel cell e.g. hydrogen/oxygen fuel cell.

| Name | Lechlanche or Dry Cell | Lead - Acid Accumulator Battery | Hydrogen/Oxygen Fuel Cell |
| --- | --- | --- | --- |
| Cathode | Graphite (C) | Lead oxide (PbO2) | Oxygen gas (O2) |
| Anode | Zinc (Zn) | Lead (Pb) | Hydrogen gas (H2) |
| Electrolyte | Manganese dioxide Ammonoium chloride | Sulfuric acid (4.5 molL−1 H2SO4) | Acid (H3PO4) Alkaline (KOH) |
| Oxidation ½ equation | Zn(s) → Zn2+(aq) + 2e− | Pb(s) + SO42−(aq) → PbSO4(s) + 2e− | H2(g) → 2H+(aq) + 2e− 2OH-(aq)+H2(g) →2H2O(l) + 2e− |
| Reduction ½ equation | 2MnO2(s) + 2H+(aq) + 2e− → Mn2O3(s) + H2O(l) | PbO2(s) + 4H+(aq) + SO42−(aq) + 2e− → PbSO4 (s) + 2H2O(l) | O2(g) + 4H+(aq) + 4e− → 2H2O(l) O2(g) + 2H2O(l) + 4e−→ 4OH-(aq) |
| Full RedOx equation | 2MnO2(s) + 2H+(aq) + Zn → Zn2+(aq) + Mn2O3(s) + H2O(l) | Discharging (e.g. starting car, leaving car lights on) Galvanic Pb(s) + PbO2(s) + 4H+(aq) + 2SO42−(aq) → 2PbSO4 (s) + 2H2O(l) | 2H2(g) + O2(g) → 2H2O(l) |
|  |  | Recharging (when car engine is running) Electolytic 2PbSO4 (s) + 2H2O(l) → Pb(s) + PbO2(s) + 4H+(aq) + 2SO42−(aq) |  |
| Cell Voltage (emf) | 1.48 V | 2.05 V per individual cell, 12 V when 6 cells connected in series. | 1.23 V – only a theoretical value |
| Advantages | Low cost, raw materials readily available Can be adapted to various shapes/designs Can be connected in series to provide greater voltages/current | Relatively low cost. Raw materials readily available Secondary cell i.e. it can be recharged, lasts approx. 5 years. Can be connected in series to provide greater voltages/current | Small mass High fuel efficiency Are direct energy converters, water is the resultant product |
| Disadvantage | Low voltage/current provided Primary cell i.e. not rechargeable Limited working life | Lead is a heavy metal poison Rather heavy due to the high density of lead. Sulfuric acid to dispose of. | Small mass, need constant input of reactants. |
| Common Uses | Torches Calculators Smoke detectors TV Remote controls | Primarily used in cars – to start the car, operate lights etc. Electric fences on a farm. Some electric powered vehicles. | Spacecraft – used as a source of electrical energy and water for astronauts |