ELECTRIC CURRENT TUTORIAL

SOLUTIONS

1.	D

2.	A

3.	B (parallel circuit)

4.	D

5.	A

6.	B

7.	B

8.	A

9.	A

10.	A

11.	B

12.	C

13.	The resistance causes a decrease in the flow of charge and a drop in electrical potential.

14.	A voltmeter measures potential difference from one point to another point.

15.	The electrical conductivity of snow is very low.

16.	The current returns to the negative side of the battery by travelling through the metal body of the car.

17.	ammeter.

18.	resistance.

19.	series.

20.	series.

21.	parallel.

22.	parallel.

23.	parallel.

24.	(a)	RT 	= 10.0 + 20.0

= 30.0 

(b) 1RT=110.0+110.0 RT=5.00 Ω(c) 1RT=15.00+110.0+110 RT=2.50 Ω(d) 1R=130.0+110.0 R=7.50 Ω RT=20.0+7.50 =27.5 Ω

(e)1R=11.00+11.00+11.00R=0.333Ω1R=1100+1100R=0.500ΩRT=0.33+1.00+0.5001.83Ω

(f)1R=11.00+12.00+13.00R=0.545Ω1R=15.00+110.0R=3.33ΩRT=0.545+4.00+3.337.88Ω

25.	V	= 	IR

240	=	0.250  R

R	=	960 

26.	V	=	IR

6.00 	=	0.100  R

R	=	60.0 

27.	(a)	RT 	=	20.0 + 30.0

=	50.0 

(b)	V	=	IR

10.0	=	I  50.0

I	=	0.200 A

(c)	V20	=	IR

=	0.200  20.0 = 4.00 V

V30	=	IR

=	0.200  30.0 = 6.00 V

28.	V	=	IR

12.0	=	0.0500  RT

RT 	=	240 

Runknown = 140 

29.	(a)	IR=16.00+118.0+19.00

RT  = 3.00 

(b)	V	=	IR

12.0	=	I  3.00

I	=	4.00 A

(c)

I6.00Ω=12.06.00=2.00AI18.0Ω=12.018.0=0.667AI9.00Ω=12.09.00=1.33A

30.	(a)	V	=	IR

240 	=	8.00 RT

RT	=	30.0 

(b)

IRT=130.0=160.0+190.0+1R3130.0−160.0−190.0=1R3R3=180Ω

31.	IR=15.00+19.001R=12.00+12.00

## R 	= 3.214 		R 	= 1.00 

RT 	= 2.00 + 3.214	RT 	= 1.00 + 5.00

= 5.214 			= 6.00 

IT=6.005.214=1.151AIT=12.06.00=2.00A

I2	= 1.15 A		I5		= 2.00 A

V2	= 1.151  2.00 = 2.30 V	V5	= 2.00  5.00 = 10.0 V

Vparallel = 6.00 – 2.30 = 3.70 V	Vparallel = 12.0 – 10.0 = 2.00 V

I3Ω,2Ω=3.705.00=0.740AI2Ω=2.002.00=1.00A

I3	= I2 = 0.740 A	V2 	= 1.00 x 2.00 = 2.00 V

V3	= 0.740  3.00 = 2.22 V

V2	= 0.740  2.00 = 1.48 V

I5Ω,4Ω=3.709.00=0.411A

I5	= I4 = 0.411 A

V5 	= 0.411  5.00 = 2.06 V

V4 	= 0.411  4.00 = 1.64 V